# PROJECT DOCUMENTATION

## Project Title

SaveSentra - IoT Family Cash Deposit System with Dynamic User Registration

## Aim

To design and implement a smart, scalable multi-user banknote validation system tailored for family savings. The device uses NFC authentication to identify individual family members before allowing deposits, with a unique dynamic registration system that allows unlimited users to be added through the Blynk mobile app without code modifications.

Unlike standard coin banks, this project utilizes a Stepper Motor for precise, constant-speed banknote intake and a Hall Effect Magnetic Encoder (AS5600) to measure the exact length of the banknote with sub-2mm precision for denomination detection.

The processed data is transmitted to the Blynk IoT Platform (FREE plan), where savings are automatically sorted and displayed under each family member's name in real-time, encouraging financial literacy, savings habits, and collective family financial goals. The system supports up to 20 family members with persistent data storage, making it ideal for extended families or small community groups.

## Objectives

### 1. Implement Secure NFC Authentication

- Deploy an RC522 NFC reader for contactless card/tag-based user identification

- Require successful authentication before allowing any deposit

- Implement 30-second authentication timeout for security

- Provide immediate visual feedback via tri-color LED indicators (Red/Blue/Green)

### 2. Enable Dynamic User Registration via Mobile App

- Design a registration interface in the Blynk mobile app where users can:

  - Enter a new family member's name (e.g., "AFTHAB")

  - Scan their NFC card on the smart box

  - The esp 32 registers the nfc card with the name from the blynk app

  - Complete registration with one button tap

- Store all user data in ESP32 flash memory for persistence across power cycles

- Support up to 20 dynamic users without requiring code changes or re-flashing

### 3. Construct a Precision Banknote Intake Mechanism

- Build a motorized dual-roller system using a 28BYJ-48 Stepper Motor

- Trigger motor activation automatically when a TCRT5000 detects note insertion

- Maintain constant intake velocity (~50mm/sec) for accurate length measurement

- Implement automatic reversal if note length exceeds maximum threshold (error detection)

### 4. Implement High-Precision Denomination Detection

- Install an AS5600 12-bit Hall Effect sensor on the free-spinning idler roller

- Mount a diametric magnet on the roller's axle for rotation tracking

- Calculate note length by converting angular displacement to linear distance

- Differentiate between UAE Dirham denominations (5, 10, 20, 50, 100 AED) with <2mm accuracy

- Reject invalid notes and eject them automatically

### 5. Develop a Comprehensive Family Dashboard

- Design a two-tab Blynk interface:

  - Dashboard Tab: Real-time display of all balances, total savings, recent transactions

  - Registration Tab: User-friendly interface for adding/managing family members

- Route deposit data to the correct user's balance based on authenticated NFC ID

- Provide push notifications for successful deposits

- Display savings progress with visual indicators (progress bars, percentages)

## Requirements

### Functional Requirements

| # | Requirement | Description |
| --- | --- | --- |
| FR-1 | User Authentication | System must authenticate users via NFC before accepting deposits. Unknown cards trigger visual error indication (red LED blink). |
| FR-2 | Dynamic Registration | Users must be able to register new NFC cards through the Blynk app without modifying ESP32 code. |
| FR-3 | Auto-Trigger Intake | Optical sensor must detect note insertion and activate stepper motor automatically within 100ms. |
| FR-4 | Length Measurement | ESP32 must calculate note length from Hall Effect sensor rotational data with ±2mm precision. |
| FR-5 | Denomination Detection | System must identify AED denominations (5/10/20/50/100) based on measured length and reject invalid notes. |
| FR-6 | Error Handling | Motor must reverse and eject notes that are too long, stuck, or invalid (timeout >8 seconds). |
| FR-7 | Balance Tracking | System must update the correct user's balance in Blynk immediately after successful deposit. |
| FR-8 | Data Persistence | User data (names, UIDs, balances) must persist in flash memory across power cycles. |
| FR-9 | Multi-User Support | System must support up to 20 registered users with independent balance tracking. |
| FR-10 | Real-Time Notifications | App must send push notifications for deposits: "AFTHAB deposited 50 AED". |
| FR-11 | Transaction Logging | System must display the last transaction details in the app. |
| FR-12 | User Management | Users must be able to view and delete registered cards through the app. |

### Non-Functional Requirements

| # | Requirement | Specification |
| --- | --- | --- |
| NFR-1 | Precision | Denomination detection accuracy: ≥98%, error margin <2mm |
| NFR-2 | Reliability | System uptime: 99% during normal operation, graceful error recovery |
| NFR-3 | Response Time | Authentication: <500ms, Deposit processing: <10 seconds |
| NFR-4 | Usability | App interface suitable for ages 7+ (child-friendly), registration in <2 minutes |
| NFR-5 | Durability | Mechanical components rated for 10,000+ note cycles |
| NFR-6 | Power Efficiency | Idle consumption: <500mA, Active: <2A (5V supply) |
| NFR-7 | Scalability | Support 20 users on FREE Blynk plan (no subscription required) |
| NFR-8 | Compatibility | Works with standard MIFARE Classic, Ultralight, NTAG NFC cards |
| NFR-9 | Network Dependency | Requires WiFi for Blynk sync; local operation continues during outages with sync on reconnection |
| NFR-10 | Maintainability | Modular code design; calibration via serial monitor without recompilation |

## System Architecture

### Data Flow Diagram

┌─────────────────────────────────────────────────────────────────┐

│                         USER INTERACTION                        │

└────────────┬────────────────────────────────────┬───────────────┘

│                                    │

▼                                    ▼

┌────────────────┐                  ┌────────────────┐

│  Registration  │                  │    Deposit     │

│   (One-time)   │                  │    (Daily)     │

└────────┬───────┘                  └────────┬───────┘

│                                    │

▼                                    ▼

┌─────────────────────────────────────────────────────┐

│              BLYNK MOBILE APP                       │

│  ┌───────────────┐        ┌─────────────────────┐  │

│  │ Registration  │        │   Main Dashboard    │  │

│  │     Tab       │        │   • Balances        │  │

│  │ • Enter name  │        │   • Transactions    │  │

│  │ • Get UID esp │        │   • Total savings   │  │

│  │ • Register    │        │                     │  │

│  └───────┬───────┘        └──────────┬──────────┘  │

└──────────┼────────────────────────────┼─────────────┘

│                            │

▼                            ▼

┌──────────────────────────────────────────────────────┐

│              BLYNK CLOUD (FREE TIER)                 │

│  				                            │

│  • User data storage & routing                       │

│  • Push notifications                                │

│  • Real-time sync                                    │

└─────────────────────┬────────────────────────────────┘

│ WiFi / Internet

▼

┌──────────────────────────────────────────────────────┐

│                  ESP32 CONTROLLER                    │

│  ┌──────────────────────────────────────────────┐   │

│  │         MAIN CONTROL LOGIC                   │   │

│  │  • User authentication                       │   │

│  │  • Motor control                             │   │

│  │  • Length calculation                        │   │

│  │  • Balance management                        │   │

│  │  • Flash storage (Preferences)               │   │

│  └──────────────────────────────────────────────┘   │

└───┬──────────┬──────────┬──────────┬────────────┬───┘

│          │          │          │            │

▼          ▼          ▼          ▼            ▼

┌────────┐ ┌────────┐ ┌────────┐ ┌────────┐ ┌──────────┐

│  NFC   │ │Optical │ │Stepper │ │ Hall   │ │   LEDs   │

│ Reader │ │ Sensor │ │ Motor  │ │ Sensor │ │  Status  │

│ RC522  │ │	        │ │28BYJ-48│ │AS5600  │ │ R/G/B    │

└────┬───┘ └────┬───┘ └────┬───┘ └────┬───┘ └──────────┘

│          │          │          │

▼          ▼          ▼          ▼

┌──────────────────────────────────────────────┐

│         PHYSICAL INTERACTION                 │

│                                              │

│  1. User taps NFC card → Authentication     │

│  2. User inserts note → Trigger detected    │

│  3. Rollers pull note → Length measured     │

│  4. Denomination identified → Balance ↑     │

└──────────────────────────────────────────────┘

### System Architecture Flow

Personalized Data Flow:

User Registration Flow:

Mobile App (Name + UID Input from nfc reader) → Blynk Cloud → ESP32 → Flash Storage → Confirmation

Deposit Flow:

NFC Card Tap → Authentication → Optical Trigger → Motor Activation →

Hall Sensor Measurement → Length Calculation → Denomination Detection →

Balance Update → Blynk Cloud → Mobile App Notification

## Hardware Requirements

### Core Components

| Component | Model/Spec | Quantity | Purpose | RECEIVED |
| --- | --- | --- | --- | --- |
| Microcontroller | ESP32 DevKit v1 (30-pin) | 1 | Main processor with WiFi | YES |
| NFC Reader | RC522 RFID Module (13.56MHz) | 1 | User authentication | YES |
| NFC Cards/Tags | MIFARE Classic 1K or NTAG215 | 5-10 | User identification tokens | YES |
| Optical Sensor | TCRT5000 | 1 | Note insertion detection | YES |
| Stepper Motor | 28BYJ-48 + ULN2003 Driver | 1 | Banknote pulling mechanism | YES |
| Hall Effect Sensor | AS5600 12-bit Magnetic Encoder | 1 | Rotation/length measurement | YES |
| Magnet | 6mm Diametric Neodymium (N52) | 1 | For AS5600 sensing | YES |
| Status LEDs | 5mm LED (Red/Green/Blue) | 3 | Visual feedback |  |
| Resistors | 220Ω (1/4W) | 3 | LED current limiting |  |
| Power Supply | 5V 2A Wall Adapter | 1 | System power |  |
| Bearings | 4mm Bore Miniature Bearings | 2 | Idler roller mounting |  |
| Rubber Tubing | Silicone 25-28mm OD | 0.5m | Roller grip surface |  |
| Enclosure Materials | 5mm Acrylic sheets + MDF base | - | Housing structure |  |
| Wiring & Misc | Jumper wires, screws, standoffs | - | Assembly |  |

## Detailed Hardware Specifications

### 1. ESP32 DevKit v1

- CPU: Dual-core Xtensa LX6 @ 240MHz

- Memory: 520KB SRAM, 4MB Flash

- Connectivity: WiFi 802.11 b/g/n, Bluetooth 4.2

- GPIO: 30 pins (26 usable)

- Interfaces: SPI, I2C, UART, ADC

- Power: 5V input via USB or VIN, 3.3V regulated output

### 2. RC522 NFC Module

- Frequency: 13.56MHz

- Protocol: ISO14443A (MIFARE)

- Read Range: 0-40mm (optimal: 20-30mm)

- Interface: SPI (up to 10Mbps)

- Supported Cards: MIFARE Classic, Ultralight, NTAG, DESFire

- Current Draw: ~13mA (active), ~10μA (standby)

### 3. Optical Sensor

- Type: TCRT5000

- Response Time: <2ms

- Output: Digital (HIGH = clear, LOW = blocked)

### 4. 28BYJ-48 Stepper Motor

- Type: Unipolar 5-wire stepper

- Voltage: 5V DC

- Step Angle: 5.625° (64 steps/revolution)

- Gear Ratio: 1:64

- Effective Steps: 2048 per full rotation

- Torque: ~34 N·cm (sufficient for note pulling)

- Speed Range: 1-15 RPM (adjustable)

### 5. AS5600 Hall Effect Sensor

- Resolution: 12-bit (4096 positions per revolution)

- Accuracy: 0.087° per step

- Interface: I2C (address 0x36)

- Update Rate: Up to 150Hz

- Magnet Requirements:

  - Type: Diametric (poles on circumference)

  - Size: 6mm diameter × 2-3mm thick

  - Distance: 0.5-3mm from sensor

- Output: Absolute angle (0-360°)

## Software Architecture

### Technology Stack

| Layer | Technology | Purpose |
| --- | --- | --- |
| Cloud Platform | Blynk IoT (FREE plan) | Data storage, mobile interface, notifications |
| Firmware | Arduino IDE (ESP32 core) | Main control logic |
| Libraries | • WiFi.h<br>• BlynkSimpleEsp32.h<br>• MFRC522.h<br>• Wire.h (I2C)<br>• Stepper.h<br>• Preferences.h | Network, NFC, sensors, storage |
| Storage | ESP32 NVS (Non-Volatile Storage) | Persistent user data |
| Communication | SPI (NFC), I2C (Hall sensor), GPIO (sensors/motor) | Hardware interfaces |

### Blynk Virtual Pin Mapping

| Virtual Pin | Data Type | Purpose | Access |
| --- | --- | --- | --- |
| V0-V19 | Double | Individual user balances (20 users) | Read/Write |
| V20 | String | Registration: User name input | Write |
| V21 | String | Registration: Card UID input | Write |
| V22 | Integer | Registration: Submit button trigger | Write |
| V23 | String | Registration: Status message output | Read |
| V24 | String | Registered users list display | Read |
| V25 | String | Last transaction details | Read |
| V26 | Double | Total family savings (sum of all balances) | Read |
| V27 | Integer | Delete user by index | Write |

### Key Algorithms

1. Length Calculation Algorithm:

1. Record initial angle when optical sensor triggers

2. Step motor continuously at constant speed

3. Read AS5600 angle

4. Detect full rotations (360° wraparound)

5. When optical sensor clears (note passed through):

total_angle = (current_angle - start_angle) + (full_rotations × 360°)

note_length = (total_angle / 360°) × wheel_circumference

2. Denomination Detection Logic:

IF length ∈ [120mm, 126mm] → 10 AED

IF length ∈ [127mm, 133mm] → 20 AED

IF length ∈ [134mm, 140mm] → 50 AED

IF length ∈ [141mm, 147mm] → 100 AED

ELSE → REJECT (reverse motor)

3. Authentication Flow:

1. Continuous NFC scanning loop

2. On card detected → Extract UID

3. Compare UID with stored users (linear search)

4. If match → Set authenticated flag, start 30s timer, LED blue

5. If no match → LED red blink 5×

6. If timeout → Reset authentication, LED red

## User Experience Flow

### Registration Process (First-Time Setup)

┌─────────────────────────────────────────────────────┐

│ STEP 1: Open SmartSave Blynk App                    │

│   1. Navigate to "Registration" tab                 │

│   2. Enter name: "AFTHAB"                           │

│   3. Send instruction to rfid reader to collect the

uid

│   4. Tap "Register User" button                     │

└─────────────────────────────────────────────────────┘

↓

┌─────────────────────────────────────────────────────┐

│ STEP 2: Confirmation                                │

│   • App shows: "✅ AFTHAB registered successfully!" │

│   • User appears in "Registered Users" list         │

│   • System ready for deposits                       │

└─────────────────────────────────────────────────────┘

Time required: ~2 minutes per user

### Daily Deposit Process

┌─────────────────────────────────────────────────────┐

│ STEP 1: Authenticate                                │

│   • User approaches SmartSave box                   │

│   • LED: RED (waiting)                              │

│   • User taps NFC card to reader zone               │

│   • LED: GREEN blink 3× → BLUE solid               │

│

└─────────────────────────────────────────────────────┘

↓

┌─────────────────────────────────────────────────────┐

│ STEP 2: Insert Banknote                             │

│   • User inserts note into slot                     │

│   • Optical sensor detects note                     │

│   • Motor activates immediately                     │

│   • Note pulled into collection chamber (2-5 sec)   │

└─────────────────────────────────────────────────────┘

↓

┌─────────────────────────────────────────────────────┐

│ STEP 3: Processing                                  │

│   • Hall sensor measures rotation                   │

│   • ESP32 calculates length: 130mm                  │

│   • Denomination identified: 20 AED                 │

│   • Balance updated: AFTHAB +20 AED                 │

└─────────────────────────────────────────────────────┘

↓

┌─────────────────────────────────────────────────────┐

│ STEP 4: Confirmation                                │

│   • LED: GREEN solid (2 seconds)                    │

│   • Push notification: "AFTHAB deposited 20 AED"    │

│   • App updates balance in real-time                │

│   • LED returns to RED (ready for next user)        │

└─────────────────────────────────────────────────────┘

Total time: ~10 seconds per deposit

## FILL EVERYTHING AFTER THIS DAILY

## Implementation Timeline

### Week 1: Mechanical Construction

Objective: Build physical enclosure and roller mechanism

| Day | Task | Deliverable |
| --- | --- | --- |
| 1-2 | Design & cut acrylic panels (or 3D print enclosure parts) | Cut panels/printed parts |
| 3 | 3D print roller cores (or purchase pre-made wheels) | 2× roller cores |
| 4 | Wrap rollers with silicone tubing, test grip | Rubberized rollers |
| 5 | Assemble roller frame with bearings, mount stepper motor | Functional roller assembly |
| 6 | Attach diametric magnet to idler axle, test alignment | Magnet mounted |
| 7 | Full mechanical assembly test with paper strips | Working intake mechanism |

Milestone: Rollers can grip and pull paper smoothly

### Week 2: Electronics Assembly

Objective: Wire and test all electronic components

| Day | Task | Deliverable |
| --- | --- | --- |
| 1 | Solder/breadboard ESP32 connections for NFC and sensors | Power-on test successful |
| 2 | Mount and align AS5600 Hall sensor (1-2mm from magnet) | Stable angle readings |
| 3 | Install U-shaped optical sensor at entry slot | Beam break detection working |
| 4 | Wire stepper motor to ULN2003 driver board | Manual motor control test |
| 5 | Connect status LEDs with current-limiting resistors | LED test (all colors) |
| 6 | Assemble complete circuit, perform continuity tests | Full circuit functional |
| 7 | Power testing under load (all components active) | Stable 5V 2A operation |

Milestone: All hardware responds correctly to test commands

### Week 3: Software Development

Objective: Write, test, and debug firmware

| Day | Task | Deliverable |
| --- | --- | --- |
| 1 | Set up Arduino IDE, install libraries, test Blynk connection | WiFi + Blynk connected |
| 2 | Write AS5600 reading code, test angle accuracy | Reliable rotation tracking |
| 3 | Calibrate wheel diameter, implement length calculation algorithm | Length measurement ±2mm |
| 4 | Write NFC authentication logic, test with 3 cards | Successful user identification |
| 5 | Integrate Blynk registration interface (V20-V24) | Dynamic user addition works |
| 6 | Implement denomination detection and balance updates | Full deposit cycle functional |
| 7 | Add flash storage (Preferences library) for persistence | Data survives power cycle |

Milestone: Complete firmware uploaded and running

### Week 4: Integration & Testing

Objective: System validation with real banknotes

| Day | Task | Deliverable |
| --- | --- | --- |
| 1 | Register 5 test users, verify data storage | 5 users in system |
| 2 | Test with real AED notes (10/20/50/100), measure accuracy | Denomination detection >95% |
| 3 | Calibrate length thresholds for each denomination | Optimized detection ranges |
| 4 | Stress test: 50 deposits, check for jams/errors | Error rate <5% |
| 5 | Test edge cases (worn notes, fast insertion, timeouts) | Graceful error handling |
| 6 | Refine Blynk dashboard UI, add progress bars | Polished user interface |
| 7 | Final enclosure assembly, cable management, labeling | Production-ready device |

Milestone: Fully functional SmartSave system ready for deployment

## Testing & Validation Plan

### Functional Testing

| Test ID | Test Case | Expected Result | Pass Criteria |
| --- | --- | --- | --- |
| FT-01 | Register new user via app | User added to system, UID stored | Confirmation message shown |
| FT-02 | Authenticate with registered card | LED blue, 30s timer starts | <500ms response time |
| FT-03 | Authenticate with unknown card | LED red blinks, no access | Rejection within 1s |
| FT-04 | Insert 10 AED note | Detected as 10 AED, balance +10 | 100% accuracy over 10 trials |
| FT-05 | Insert 50 AED note | Detected as 50 AED, balance +50 | 100% accuracy over 10 trials |
| FT-06 | Insert oversized paper | Motor reverses, note ejected | No jam, paper returned |
| FT-07 | Authentication timeout | LED returns to red after 30s | Precise timing |
| FT-08 | Power cycle test | All user data restored | Zero data loss |
| FT-09 | Concurrent deposits | Only authenticated user credited | Correct account routing |
| FT-10 | Delete registered user | User removed, card no longer works | Immediate effect |

### Performance Testing

| Metric | Target | Measurement Method |
| --- | --- | --- |
| Denomination Accuracy | ≥95% | 100 deposits across all denominations |
| Length Measurement Precision | ±2mm | Compare to caliper measurements (n=50) |
| Authentication Speed | <500ms | Timestamp NFC read to LED response |
| Deposit Processing Time | <10s | Entry to balance update completion |
| Motor Jam Rate | <2% | 200 note insertions with varying conditions |
| WiFi Reconnection | <30s | Disconnect router, measure reconnect time |
| Flash Write Cycles | 10,000+ | Stress test registration/deletion loops |

### Edge Case Testing

- Crumpled/damaged notes: Test with intentionally folded banknotes

- Rapid consecutive deposits: Insert 5 notes within 2 minutes

- Simultaneous card taps: Two users tap cards at same time

- Network outages: Disconnect WiFi during deposit, verify local caching

- Low battery scenarios: Test with 4.5V supply (undervoltage)

- Temperature extremes: Operate at 10°C and 35°C ambient

## Cost Breakdown & Budget

### Hardware Costs

| Category | Items | Subtotal |
| --- | --- | --- |
| Electronics | ESP32, RC522, AS5600, sensors, LEDs, wiring | $25 |
| Mechanical | Stepper motor, bearings, magnet, rollers | $13 |
| Enclosure | Acrylic, MDF, screws, standoffs | $15 |
| Power | 5V 2A adapter | $5 |
| NFC Cards | 10× MIFARE Classic tags | $5 |
| TOTAL HARDWARE |  | $63 |

### Total Project Cost: ~$63

## Conclusion

The SmartSave IoT Family Cash Deposit System successfully combines mechanical precision, sensor technology, and cloud connectivity to create an educational and practical tool for family financial management.

Key Innovations:

- Dynamic user registration via mobile app (no hardware reprogramming)

- High-precision denomination detection using Hall Effect sensing (<2mm accuracy)

- Scalable architecture supporting 20 users on a FREE cloud platform

- Child-friendly interface encouraging savings habits from age 7+

Educational Impact:

- Teaches children about money management and delayed gratification

- Provides real-time visibility into family savings progress

- Gamifies the savings process with visual progress indicators

- Promotes financial literacy through hands-on interaction

Technical Achievement:

- Demonstrates integration of NFC, stepper motors, magnetic encoders, and IoT

- Showcases ESP32 capabilities (WiFi, multi-sensor fusion, flash storage)

- Implements robust error handling and edge-case management

- Achieves professional-grade accuracy with consumer-grade components

Total Development Time: 4 weeks
Total Budget: ~$63 (hardware only)
Recurring Costs: $0 (FREE Blynk plan)

This project serves as both a functional family savings tool and a comprehensive learning experience in embedded systems, mechanical design, and IoT application development.

## Appendix

### A. Component Datasheets

- ESP32 DevKit v1: Espressif Datasheet

- RC522 NFC Module: NXP MFRC522 Datasheet

- AS5600 Hall Sensor: AMS Datasheet

- 28BYJ-48 Stepper: Motor Specifications

### B. 3D Printable Files (To Be Generated)

- Roller cores (STL)

- Motor mounting bracket (STL)

- AS5600 sensor holder (STL)

- Bearing blocks (STL)

- Enclosure assembly jigs (STL)

### C. Blynk Template QR Code

[Generate after template creation in Blynk Console]

### D. User Manual (Quick Start)

[Create simplified 1-page guide for family members]

### E. GitHub Repository

[Code, schematics, and documentation to be hosted]

Project Author: [Your Name]
Institution: [Your University/Organization]
Date: January 2026
Version: 1.0

This project is designed for educational purposes and family use. For commercial applications, please ensure compliance with local electronic device regulations and payment system laws.