## 🔹 DAY 1 – SYSTEM SETUP (FOUNDATION DAY)

☐ Install Arduino IDE + ESP32 board manager
☐ Install libraries:

- Blynk

- MFRC522

- Preferences

- Wire

- Stepper (or AccelStepper – better)
☐ Create Git folder / backup zip (yes, do it)
☐ Write empty sketch that compiles for ESP32

Goal: Zero errors. Clean environment.

## 🔹 DAY 2 – NFC ONLY (NO BLYNK YET)

☐ Wire RC522 to ESP32 (SPI)
☐ Upload basic NFC UID reader
☐ Print UID to Serial Monitor
☐ Test 5 times with same card (consistency)

Tick only if:
You can tap → UID prints instantly (<500ms)

## 🔹 DAY 3 – NFC + FLASH STORAGE

☐ Store UID in Preferences
☐ Store a dummy name (“TEST_USER”)
☐ On boot → print stored users
☐ Verify data persists after reset

Goal:
You understand how your data lives. No magic.

## 🔹 DAY 4 – STEPPER MOTOR CONTROL (ISOLATED)

☐ Wire ULN2003 + 28BYJ-48
☐ Rotate motor forward
☐ Rotate backward
☐ Fix speed (~50 mm/s equivalent)

⚠️ If motor skips or stalls → FIX NOW
Not later. Not tomorrow.

## 🔹 DAY 5 – OPTICAL SENSOR (TCRT5000)

☐ Wire TCRT5000
☐ Print HIGH/LOW on Serial
☐ Trigger using paper note
☐ Debounce logic (simple delay ok)

Condition:
Sensor must trigger before motor starts

## 🔹 DAY 6 – AS5600 (MOST IMPORTANT DAY)

☐ Wire AS5600 via I2C
☐ Read angle values
☐ Rotate magnet by hand
☐ Confirm 0–4095 wraparound

If this day fails → project is dead.
No AS5600 = no uniqueness.

## 🔹 DAY 7 – LENGTH MEASUREMENT TEST

☐ Mount magnet + roller
☐ Rotate roller manually
☐ Calculate distance = angle → length
☐ Measure using ruler → compare

🎯 Acceptable error: ±3 mm
Don’t be a perfectionist idiot.

# WEEK 2 = INTEGRATION (THIS IS WHERE PEOPLE FAIL)

## 🔹 DAY 8 – MOTOR + AS5600 TOGETHER

☐ Motor runs
☐ AS5600 tracks rotation live
☐ Count full rotations
☐ Print live length

Rule:
No Blynk. No NFC. One thing only.

## 🔹 DAY 9 – TCRT5000 → MOTOR AUTO START

☐ Insert note
☐ Motor starts
☐ Remove note
☐ Motor stops

If this isn’t rock solid → DO NOT PROCEED.

## 🔹 DAY 10 – FULL MEASUREMENT PIPELINE

☐ NFC → Authenticate
☐ Insert note
☐ Measure length
☐ Print denomination to Serial

Still no Blynk.
Still no LEDs.

## 🔹 DAY 11 – DENOMINATION LOGIC

☐ Hardcode UAE note ranges
☐ Test with:

- Short paper

- Long paper

- Correct note
☐ Reverse motor on invalid

🎯 Examiner LOVES rejection logic.

## 🔹 DAY 12 – FLASH BALANCE STORAGE

☐ Create user struct:

- UID

- Name

- Balance
☐ Update balance after deposit
☐ Verify persistence after reboot

## 🔹 DAY 13 – BLYNK BASIC DASHBOARD

☐ Connect ESP32 to Blynk
☐ Show:

- One balance

- Total balance
☐ Push notification on deposit

If Blynk fails → simplify UI, not logic.

## 🔹 DAY 14 – DYNAMIC USER REGISTRATION

☐ Name input from app
☐ Tap NFC
☐ Save UID + name
☐ Display list in app

⚠️ Max 20 users. Hard limit. Done.

# WEEK 3 = POLISH + DEMO PREP

## 🔹 DAY 15 – LED STATUS LOGIC

☐ RED = Idle
☐ BLUE = Authenticated
☐ GREEN = Success
☐ RED blink = Error

Simple. Clear. Examiner-friendly.

## 🔹 DAY 16 – FULL SYSTEM TEST

☐ Register 2 users
☐ Deposit notes
☐ Wrong card test
☐ Invalid note test
☐ Power cut → reboot → verify data

## 🔹 DAY 17 – MECHANICAL CLEANUP

☐ Align rollers
☐ Reduce friction
☐ Secure AS5600 & magnet
☐ Cable management

Don’t make it pretty.
Make it reliable.

## 🔹 DAY 18 – ENCLOSURE (LAST)

☐ Cut slots
☐ Mount components
☐ Ensure NFC access
☐ Easy note insertion

## 🔹 DAY 19 – DOCUMENTATION CLEANUP

☐ Architecture diagram
☐ Flow explanation
☐ Why AS5600 > ultrasonic
☐ Why NFC > PIN

## 🔹 DAY 20 – VIVA PREP (CRITICAL)

You must answer:

- Why AS5600?

- Why stepper not DC?

- How do you ensure accuracy?

- What happens on power failure?

- What would you improve with more time?

## 🔹 DAY 21 – FINAL DEMO RUN

☐ Clean table
☐ Calm explanation
☐ Slow demo
☐ Don’t oversell